USER DOCUMENTATION  -  Version 2.0

Programs :  INVERSE, FORWARD, INVERS3D, FORWRD3D

Introduction :

The geodetic inverse, program INVERSE, is the tool for computing

the geodetic azimuth and distance between two points, given their

latitudes and longitudes.

The geodetic forward, program FORWARD, is the tool for computing

the latitude and longitude of a point, given the latitude and

longitude of a starting point and the geodetic azimuth and geodetic

distance between the points.

The three-dimensional versions of the geodetic inverse and

geodetic forward, programs INVERS3D and FORWRD3D, include the

ellipsoid height component.  Also included in the 3D programs are:

1) Conversions between geodetic positions (latitude, longitude,

and ellipsoid height) and rectangular coordinates (X,Y,Z in the

Conventional Terrestrial Reference System).

2) Conversions between the following four observation sets:

a) DN,DE,DU (differential N, E, U using the FROM station

as the origin of the NEU-coordinate system.)

b) DX,DY,DZ (differential X, Y, Z used to express GPS vectors)

c) delta height, forward azimuth, ellipsoidal distance

d) delta height, forward azimuth, mark-to-mark distance

Hardware Requirements :

Programs INVERSE, FORWARD, INVERS3D, and FORWRD3D are all written

in Fortran77 and complied on an IBM-PC or compatible.

- A math-coprocessor is required,

- 256 kb of RAM, and

- DOS version 2.1 or greater.

User Instructions :

All four programs run interactively; prompts are given for input,

and the output is directed to the screen.  Simply type the name

of the desired program, then answer the prompts as they appear.

Once all the prompts have been answered, the output will appear

on the screen.  A prompt is given for saving the output into a

file, if desired.

Programs INVERSE and FORWARD allow the user to select from the

following list of reference ellipsoids for the computations:

Clarke 1866    (NAD27)

GRS80 / WGS84  (NAD83)

Airy 1858

Airy Modified

Australian National

Bessel 1841

Clarke 1880

Everest 1830

Everest Modified

Fisher 1960

Fisher 1968

Hough 1956

International (Hayford)

Krassovsky 1938

NWL-9D (WGS 66)

South American 1969

Soviet Geod. System 1985

WGS 72

User defined.

Please note that the GRS80 and WGS84 are considered to be the

same.  Actually, there is a very small difference in the

flattening which results in the semi-minor axis, b, being

different by 0.0001 meters.  There is no known application for

which this difference is significant.

The user is given the option of defining their own reference

ellipsoid in whatever units they prefer.  (If, for example, the

user wishes to define the semi-major and semi-minor axes in

miles, they may do so.  The units for the geodetic distance would

then be given in miles also.)  The units for all the listed

reference ellipsoid are given in meters, and therefore the

distances are also in meters.

Programs INVERS3D and FORWRD3D work exclusively on the GRS80

ellipsoid and the units are meters.

Note 1 :

Program FORWRD3D will do a conversion from mark-to-mark distance

to ellipsoidal distance, if a mark-to-mark distance is given as

input.  The first approximation is computed using formulas given

in the NOAA Technical Memorandum NOS NGS-10, Use of Calibration

Baselines, pages 26-27.  These formulas are generally good for

distances less than 100 kilometers.  The program will then do a

forward computation using the computed ellipsoidal distance.  The

mark-to-mark distance is then recomputed from the resulting

DX,DY,DZ.  If the difference between the computed mark-to-mark

distance and the original mark-to-mark distance is greater than

one millimeter, then the difference is applied to the ellipsoidal

distance and another forward computation is performed.  The

program will continue iterating in this fashion until convergence.

Note 2 :

All four program read the input geodetic positions as real

numbers (latitudes are positive North, and longitudes are

positive East), then convert them to radians for internal

computations.  The conversion is done as follows:

radians = ( pi / 180 )* (degrees + minutes/60 + seconds/3600)

The programs return the input values after :

1. Converting them into degrees, minutes, and seconds and

2. Checking d,m,s for the correct within range.

Some example latitude inputs are :

Hemisphere h = (blank,N,n,+   or  S,s,-)

Commas or blanks may be used between the D,M,S entries.

Keyed Input :                              Converted latitude :

Nominal : degrees, minutes & seconds ( hDD MM SS.sssss )

=========

hDD MM SS.sssss                            Latitude :

<cr>                   ( h default = N )     0  0  0.00000 North

0                             "                     "

0 0 0                         "                     "

0,0,0.0                       "                     "

0d 0m 0s                      "                     "

00d 00m 00.000s               "                     "

10 0 1.00001                  "             10  0  1.00001 North

10d 0m 1.00001s               "             10  0  1.00001 North

+0                                           0  0  0.00000 North

+0 0 0.0                                     0  0  0.00000 North

+0,0,0.1                                     0  0  0.10000 North

N0                                           0  0  0.00000 North

N20                                         20  0  0.00000 North

n20d20m22s                                  20 20 22.00000 North

n020d  20m 0022s                            20 20 22.00000 North

n10 0 10.00001                              10  0 10.00001 North

-0 0 1.00001                                 0  0  1.00001 South

-10 0 10.00001                              10  0 10.00001 South

S26 37 48.26371                             26 37 48.26371 South

Packed : degrees-minutes-seconds ( hDDMMSS.sssss )

========

hDDMMSS.sssss                              Latitude :

N000000.000                                  0  0  0.00000 North

N100010.00001                               10  0 10.00001 North

S263748.26371                               26 37 48.26371 South

Decimal : degrees ( hDD.dddddddd )

=========

hDD.dddddddd                               Latitude :

20.0                   ( h default = N )    20  0  0.00000 North

10.002777781                  "             10  0 10.00001 North

s26.630073253                               26 37 48.26371 South

=======================================================================

Some example longitude inputs are :

Hemisphere h = (E,e,+   or  blank,W,w,-)

Commas or blanks may be used between the D,M,S entries.

Keyed Input :                              Converted longitude :

Nominal : degrees, minutes & seconds ( hDDD MM SS.sssss )

=========

hDDD MM SS.sssss                           Longitude :

<cr>                   ( h default = W )     0  0  0.00000 West

0                             "                     "

0 0 0                         "                     "

0,0,0.0                       "                     "

0d 0m 0s                      "                     "

00d 00m 00.000s               "                     "

10 0 1.00001                  "             10  0  1.00001 West

10d 0m 1.00001s               "             10  0  1.00001 West

-1                                           1  0  0.00000 West

-0 0 1.00001                                 0  0  1.00001 West

-10 0 10.00001                              10  0 10.00001 West

W0                                           0  0  0.00000 West

W20                                         20  0  0.00000 West

w20d20m22s                                  20 20 22.00000 West

w020d  20m 0022s                            20 20 22.00000 West

w10 0 10.00001                              10  0 10.00001 West

+0                                           0  0  0.00000 East

+0 0 0.0                                     0  0  0.00000 East

+0,0,0.1                                     0  0  0.10000 East

E26 37 48.26371                             26 37 48.26371 East

Packed : degrees-minutes-seconds ( hDDDMMSS.sssss )

========

hDDDMMSS.sssss                             Longitude :

W0000000.000                                 0  0  0.00000 West

W0100010.00001                              10  0 10.00001 West

E0263748.26371                              26 37 48.26371 East

Decimal : degrees ( hDDD.dddddddd )

=========

hDDD.dddddddd                              Longitude :

20.0                   ( h default = W )    20  0  0.00000 West

10.002777781                  "             10  0 10.00001 West

-10.002777781                               10  0 10.00001 West

+10.002777781                               10  0 10.00001 East

e26.630073253                               26 37 48.26371 East

=======================================================================

Sample Outputs From Programs INVERSE and FORWARD

Ellipsoid : GRS80 / WGS84  (NAD83)

Equatorial radius,  a   =    6378137.0000

Polar radius,       b   =    6356752.3141

Inverse flattening, 1/f =  298.25722210088

First  Station : Jones

----------------

LAT =  34  0 12.12345 North

LON = 111  0 12.12345 West

Second Station : Smith

----------------

LAT =  33 22 11.54321 North

LON = 112 55 44.33333 West

Forward azimuth        FAZ = 249  3 16.4237 From North

Back azimuth           BAZ =  67 59 11.1619 From North

Ellipsoidal distance     S =    191872.1190 m

=================================================

Ellipsoid : Clarke 1866    (NAD27)

Equatorial radius,  a   =    6378206.4000

Polar radius,       b   =    6356583.8000

Inverse flattening, 1/f =  294.97869821380

First  Station : charlie

----------------

LAT =  45  0 12.00000 North

LON =  68  0  0.00000 West

Second Station : sam

----------------

LAT =  44 33  0.00000 North

LON =  70 12 34.78900 West

Forward azimuth        FAZ = 254 42 44.6439 From North

Back azimuth           BAZ =  73  9 21.3315 From North

Ellipsoidal distance     S =    182009.1679 m

=================================================

Sample Inputs

FROM : bill

LAT : 34,00,00.0

LON : 111 00 00.0

EHT : 76.0

dh : -22.0

FAZ : 245,00,34.7

S : 123456.7891

TO : george

Sample Outputs From Program FORWRD3D

First  Station : bill

----------------

X =   -1896956.7156 m  LAT =  34  0  0.00000 North

Y =   -4941741.1960 m  LON = 111  0  0.00000 West

Z =    3546489.0623 m  EHT =        76.0000  Meters

Second Station : george

----------------

X =   -2011507.3910 m  LAT =  33 31 25.93490 North

Y =   -4927927.6718 m  LON = 112 12 16.40986 West

Z =    3502571.6004 m  EHT =        54.0000  Meters

Forward azimuth        FAZ = 245  0 34.7000 From North

Back azimuth           BAZ =  64 20 24.6862 From North

Ellipsoidal distance     S =    123456.7891 m

Delta height            dh =       -22.0000 m

Mark-to-mark distance    D =    123456.1224 m

DX =   -114550.6754 m   DN =    -52153.4448 m

DY =     13813.5242 m   DE =   -111892.5928 m

DZ =    -43917.4619 m   DU =     -1216.5690 m

=================================================

Sample Inputs

FROM : MOUNT WHITNEY, CA

LAT : 36,34,42.89133

LON : 118 17 31.18182

EHT : 4395.832

TO : BAD WATER (DEATH VALLEY), CA

LAT : 36 1 37

LON : 116,49,32

EHT : -101.868

Sample Output From Programs INVERS3D

First  Station : MOUNT WHITNEY, CA

----------------

X =   -2432166.7206 m  LAT =  36 34 42.89133 North

Y =   -4518536.0565 m  LON = 118 17 31.18182 West

Z =    3782560.2447 m  EHT =      4395.8320  Meters

Second Station : BAD WATER (DEATH VALLEY), CA

----------------

X =   -2330458.8019 m  LAT =  36  1 37.00000 North

Y =   -4608415.6501 m  LON = 116 49 32.00000 West

Z =    3730550.0939 m  EHT =      -101.8680  Meters

Forward azimuth        FAZ = 114 29 26.9586 From North

Back azimuth           BAZ = 295 21 32.6566 From North

Ellipsoidal distance     S =    145239.0603 m

Delta height            dh =     -4497.7000 m

Mark-to-mark distance    D =    145354.3872 m

DX =    101707.9187 m   DN =    -60202.3026 m

DY =    -89879.5936 m   DE =    132158.0537 m

DZ =    -52010.1508 m   DU =     -6150.5675 m

Zenith (mk-to-mk)       ZD =  92 25 30.56

Apparent zenith distance   =  92 19 55.21

=================================================